- Calibrating the Force Sensor: Determining Relationships between Applied Force and Sensor Resistance/Conductance INSTRUCTIONS

FlexiForce sensor

two 6-inch wires (22 gauge)

weight set

wood puck

multimeter

Use this instruction manual to help you connect the force sensor to the multimeter. Place a wooden puck and weights on top of the sensor to obtain a sensor reading. The goal is to determine the relationships between the applied force and sensor output. Make sure you have the following items:

- NOTE: Record all calibration data on the Calibrating the Force Sensor Data Collection Sheet.

- Instructions

- FlexiForce Sensor Pins

Figure 1

  - The middle pin is non-active.

  - Connect wires at FlexiForce sensor pin 1 and pin 3.

Make sure that pin 1 DOES NOT TOUCH pin 3.

  - To do this, use tape, solder or a pin connector.

- Breadboard Connection

  - Connect sensor pin 3 to ground (black) on the multimeter.

  - Connect sensor pin 1 to resistance (red) on the multimeter.

  - Set your multimeter to 20M ohms.

Figure 2A-B

Figure 2C

- Puck

Place a wooden puck on the center of your sensor.

- A puck is an object that has the same diameter as your sensing area.

- A puck is used when the force area being applied is greater than your sensing area.

Figure 3

- Weights

Place a weight on the center of your sensor. Make sure the weight is within your sensor weight max.

  - Record the force and resistance values on the Calibrating the Force Sensor Data Collection Sheet.

  - Repeat with different weight values.

Figure 4A

Figure 4B-1

Figure 4B-2